| EX NO: 8a | SIMULATION AND ANALYSIS OF BPSK MODULATION USING CONSTELLATION DIAGRAMS IN MATLAB |
| --- | --- |
| DATE: |  |

Objective

i) To generate BPSK symbols from a binary data sequence using phase-shift keying principles.

ii) To Plot and analyze the BPSK signal constellation in the In-Phase (I) and Quadrature (Q) plane.

iii) To investigate the effect of noise on constellation points and evaluate the resulting degradation in symbol detection performance.

Matlab code

clc;

clear;

close all;

%% Objective 1: Generate BPSK symbols

N = 1000;                      	% Number of bits

data = randi([0 1], N, 1);     	% Random binary sequence

% BPSK Mapping

bpsk_symbols = 2*data - 1;

%% Objective 2: Plot BPSK Signal Constellation

figure;

subplot(1,2,1);

scatter(real(bpsk_symbols), imag(bpsk_symbols), ...

30, 'filled');

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlim([-1.5 1.5]);

ylim([-1 1]);

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title('Ideal BPSK Constellation');

%% Objective 3: Add Noise and Evaluate Performance

SNR = 5;                       	% Signal-to-Noise Ratio (dB)

rx_signal = awgn(bpsk_symbols, SNR, 'measured');

subplot(1,2,2);

scatter(real(rx_signal), imag(rx_signal), ...

15, 'filled');

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlim([-2.5 2.5]);

ylim([-2 2]);

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title(['Noisy BPSK Constellation (SNR = ', num2str(SNR), ' dB)']);

%% Symbol Detection

detected_bits = real(rx_signal) > 0;

% Calculate Errors

num_errors = sum(data ~= detected_bits);

BER = num_errors/N;

fprintf('Number of Transmitted Bits : %d\n', N);

fprintf('Number of Bit Errors   	: %d\n', num_errors);

fprintf('Bit Error Rate (BER)   	: %f\n', BER);

Procedure:

Objective 1: Generate BPSK Symbols from a Binary Data Sequence Using Phase-Shift Keying Principles

- Initialize the MATLAB environment and define the number of bits.

- Generate a random binary sequence consisting of 0s and 1s.

- Apply BPSK modulation by mapping binary 0 to −1 and binary 1 to +1.

- Store the generated BPSK symbols for further processing.

- Verify that the symbol sequence contains only two distinct symbol values.

- Prepare the modulated symbols for constellation analysis.

OUTPUT:

Fig: Signal Constellation for BPSK Symbols

Result BPSK symbols were successfully generated from the binary data sequence using phase-shift keying modulation.

.

Objective 2: Plot and Analyze the BPSK Signal Constellation in the In-Phase (I) and Quadrature (Q) Plane

- Use the generated BPSK symbols as inputs for constellation plotting.

- Assign the real part of the symbols to the In-Phase (I) axis.

- Set the Quadrature (Q) component to zero for all symbols.

- Plot the constellation points on the I–Q plane using a scatter plot.

- Draw the horizontal and vertical axes through the origin and label them appropriately.

- Analyze the constellation diagram and identify the symbol locations corresponding to binary 0 and binary 1.

Fig. 2: Noisy BPSK Constellation

Result : The BPSK constellation was successfully plotted in the In-Phase (I) and Quadrature (Q) plane, showing two distinct symbol points corresponding to binary 0 and binary 1 for the given SNR.

| EX NO: 8b | SIMULATION AND ANALYSIS OF QPSK MODULATION USING CONSTELLATION DIAGRAMS IN MATLAB |
| --- | --- |
| DATE: |  |

Objective

i) To generate QPSK symbols from a binary data sequence using quadrature phase-shift keying principles.

ii) To plot and analyze the QPSK signal constellation in the In-Phase (I) and Quadrature (Q) plane.

iii) To investigate the effect of noise on constellation points and evaluate the resulting degradation in symbol detection performance.

Matlab code

clc;

clear;

close all;

%% Objective 1: Generate QPSK Symbols

N = 1000;                      	% Number of bits

data = randi([0 1], N, 1);     	% Random binary sequence

% Ensure even number of bits

if mod(N,2)~=0

data = [data; 0];

N = N + 1;

end

% Group bits into pairs

bit_pairs = reshape(data,2,[])';

% QPSK Mapping

qpsk_symbols = zeros(size(bit_pairs,1),1);

for k = 1:size(bit_pairs,1)

if isequal(bit_pairs(k,:),[0 0])

qpsk_symbols(k) = (1+1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[0 1])

qpsk_symbols(k) = (-1+1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[1 1])

qpsk_symbols(k) = (-1-1j)/sqrt(2);

elseif isequal(bit_pairs(k,:),[1 0])

qpsk_symbols(k) = (1-1j)/sqrt(2);

end

end

%% Objective 2: Plot QPSK Constellation

scatter(real(qpsk_symbols),imag(qpsk_symbols), 30);

xlim([-1 1])

ylim([-1 1])

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title('Ideal QPSK Constellation');

%% Objective 3: Add Noise and Evaluate Performance

SNR = 5;

rx_signal = awgn(qpsk_symbols,SNR,'measured');

figure;

scatter(real(rx_signal),imag(rx_signal), 15);

hold on;

xline(0,'k','LineWidth',1.5);

yline(0,'k','LineWidth',1.5);

grid on;

axis equal;

xlabel('In-Phase (I)');

ylabel('Quadrature (Q)');

title(['Noisy QPSK Constellation (SNR = ',num2str(SNR),' dB)']);

%% Symbol Detection

detected_bits = zeros(length(rx_signal)*2,1);

for k = 1:length(rx_signal)

I = real(rx_signal(k));

Q = imag(rx_signal(k));

if I>0 && Q>0

bits = [0 0];

elseif I<0 && Q>0

bits = [0 1];

elseif I<0 && Q<0

bits = [1 1];

else

bits = [1 0];

end

detected_bits(2*k-1:2*k) = bits;

end

%% BER Calculation

num_errors = sum(data ~= detected_bits);

BER = num_errors/N;

fprintf('Number of Transmitted Bits : %d\n',N);

fprintf('Number of Bit Errors   	: %d\n',num_errors);

fprintf('Bit Error Rate (BER)   	: %f\n',BER);

Objective (i): Generate QPSK Symbols from a Binary Data Sequence Using Quadrature Phase-Shift Keying Principles

Procedure:

- Initialize the MATLAB environment and define the number of bits.

- Generate a random binary sequence consisting of 0s and 1s.

- Group the binary sequence into pairs of bits.

- Apply QPSK modulation by mapping each bit pair to a unique constellation point.

- Generate the corresponding complex QPSK symbols.

- Verify that four distinct symbol values are obtained.

- Store the generated symbols for constellation analysis.

OUTPUT GRAPH

Fig. 1: Signal Constellation for QPSK Symbols

Result

QPSK symbols were successfully generated from the binary data sequence using quadrature phase-shift keying modulation.

Objective (ii): Plot and Analyze the QPSK Signal Constellation in the In-Phase (I) and Quadrature (Q) Plane

Procedure

- Use the generated QPSK symbols for constellation plotting.

- Extract the real component as the In-Phase (I) coordinate.

- Extract the imaginary component as the Quadrature (Q) coordinate.

- Plot the constellation points on the I–Q plane using a scatter plot.

- Draw the horizontal and vertical axes through the origin.

- Identify the four symbol locations corresponding to the four possible bit pairs.

- Analyze the distribution of symbols in the four quadrants.

Fig. 2: Noisy QPSK Constellaion

Result

The QPSK constellation was successfully plotted in the In-Phase (I) and Quadrature (Q) plane, showing four distinct symbol points corresponding to the bit pairs 00, 01, 11, and 10.